Brainco RevoLimb Hand Integration

Gear-Sonic — Change Log & Deployment Guide

PICO XRoboToolkit hand-tracking → dex_retargeting → brainco DDS
Status: software-validated; pending hardware bring-up (Stages 2–4)

# 1. Executive summary

This release adds Brainco RevoLimb dexterous-hand control to the Gear-Sonic teleop stack. The PICO headset's OpenXR hand-tracking stream now drives both:

the existing arm IK pipeline, replacing button-encoded synthetic fingertip transforms with real anatomical fingertip positions; and

a new parallel pipeline that retargets the hand pose to brainco joint angles and publishes them on the rt/brainco/{left,right}/cmd DDS topics.

All software-only validation passes (compile, 18 unit/e2e tests, smoke test). Hardware-side validation requires a connected PICO and powered brainco hands and is described in §6 of this document.

# 2. What changed

## 2.1 New module: decoupled_wbc...pico.brainco

| File | Purpose |
| --- | --- |
| __init__.py | Lazy-loads BraincoController so dev environments without unitree_sdk2py can still import the bridge math. |
| brainco_bridge.py | Pure-NumPy frame transform: xrobotoolkit (26, 7) hand state → (25, 3) keypoint cloud in unitree-hand frame; shared-array helpers. |
| hand_retargeting.py | BraincoHandRetargeting wrapper around dex_retargeting. Normalises the union-style brainco.yml (DexPilot/vector key suffixes) into the canonical RetargetingConfig shape. |
| robot_hand_brainco.py | BraincoController: subprocess control loop that retargets, normalises to [0, 1] per the brainco API, and publishes on DDS. Includes init timeout, warmup gate, and tracking-loss revert. |
| assets/brainco_hand/ | URDFs (left + right), STL meshes, and brainco.yml — all copied verbatim from xr_teleoperate. |
| README.md | In-tree integration doc with derivation, runbook, and failure-mode table. |

## 2.2 New runners and tests

| File | Purpose |
| --- | --- |
| control/teleop/main/run_brainco_teleop.py | Standalone CLI (no arm IK), exposes --state-timeout-s and --warmup-frames flags. |
| control/teleop/main/smoke_test_brainco.py | Hardware-free pre-deploy check: imports + dex_retargeting URDF wiring + bridge math on synthetic input. |
| tests/control/teleop/brainco/test_brainco_bridge.py | 14 unit tests covering rotation correctness, palm-drop, frame anchor, and shared-array roundtrip. |
| tests/control/teleop/brainco/test_brainco_retarget_e2e.py | 4 e2e tests, gated on dex_retargeting being installed: YAML loads, retarget runs, output sensitivity, motor-angle ranges. |

## 2.3 Modified files

| File | Change |
| --- | --- |
| decoupled_wbc/control/teleop/streamers/pico_streamer.py | Constructor gained enable_brainco, brainco_fps, brainco_state_timeout_s, brainco_warmup_frames. _generate_finger_data now fills (25, 4, 4) with real wrist-anchored hand keypoints (in meters). Falls back to button-encoded synthetic signal only when hand-tracking is_active=0. |
| decoupled_wbc/control/teleop/device/pico/xr_client.py | Hand-tracking docstring corrected from 27×7 to 26×7, with the Khronos OpenXR XR_EXT_hand_tracking joint enum and pose convention spelled out. |
| gear_sonic/pyproject.toml | Added dex-retargeting and pyyaml to the [teleop] optional-dependency group, so install_pico.sh picks them up automatically. |

# 3. Why these changes

The primary motivation is replacing fake fingertip data with real hand tracking. The previous _generate_finger_data path encoded trigger/grip button combos as synthetic (1, 0, 0) fingertip transforms — useful as a pinch fallback but blind to the user's actual fingers. With hand tracking already available from xrobotoolkit, the synthetic path was wasting a real signal.

The second motivation is enabling Brainco RevoLimb hand control. Reusing the same xrobotoolkit hand state for both pipelines avoids a second SDK connection and keeps the data flow auditable.

## 3.1 Data flow

PICO headset (OpenXR app)
  └─ XRoboToolkit-PC-Service
       └─ xrobotoolkit_sdk
            └─ XrClient.get_hand_tracking_state()        [main process]
                 ├─► hand_state_to_unitree_keypoints()  → (25, 3) keypoints
                 │      │
                 │      ├─► push_keypoints_to_shared()  → Array('d', 75)
                 │      │      │
                 │      │      └─► BraincoController._control_loop  [subprocess]
                 │      │             dex_retargeting → normalise → DDS
                 │      │             rt/brainco/{left,right}/cmd
                 │      │
                 │      └─► fingertips[:, :3, 3]   → (25, 4, 4)
                 │             │
                 │             └─► G1GripperInverseKinematicsSolver
                 │                    thumb-to-tip distances → arm pose
                 └─► Brainco RevoLimb hands (via DDS)

## 3.2 Coordinate-frame correctness

dex_retargeting expects wrist-relative finger vectors in the brainco URDF base_link frame. We arrive there from xrobotoolkit's OpenXR hand-tracking output via a single composed rotation:

R_HAND_TO_UNITREE = T_TO_UNITREE_HAND @ R_ROBOT_OPENXR
                  = [[0, 1, 0],
                     [0, 0, 1],
                     [1, 0, 0]]

delta_unitree = R_HAND_TO_UNITREE @ wrist_R_openxr.T @ (tip - wrist)_world

Both T_TO_UNITREE_HAND and R_ROBOT_OPENXR are copied verbatim from xr_teleoperate's televuer pipeline. The composed matrix is orthogonal with det = 1 (verified in test_R_HAND_TO_UNITREE_*). The full algebraic derivation is in the in-tree README §6.

# 4. Safety features

BraincoController has three runtime guards. Each is exposed as a kwarg on BraincoController and PicoStreamer, and as a CLI flag on run_brainco_teleop.py.

| Guard | Default | Behaviour |
| --- | --- | --- |
| DDS state-init timeout (state_timeout_s) | 5.0 s | If rt/brainco/{left,right}/state isn't publishing, raises TimeoutError instead of hanging the streamer forever. Set to 0 to wait forever (legacy). |
| Warmup gate (warmup_frames) | 10 frames | First N consecutive valid input frames are NOT retargeted; controller publishes q=0 (open hand). Blocks cold-start xrobotoolkit junk from slamming the hand closed. |
| Tracking-loss revert | always on | If input becomes all-zero or non-finite after warmup, controller drops back to open hand and re-arms the warmup counter. One noisy frame costs you warmup_frames/fps seconds of "open hand" but eliminates the slam-shut failure mode. |

# 5. Validation status (what's done)

| Stage | Validation | Status |
| --- | --- | --- |
| 0 | Every brainco source file compiles cleanly (ast.parse + compile, no warnings). | PASS |
| 1a | dex_retargeting can build BraincoHandRetargeting from the in-tree URDFs and brainco.yml. Verified with python-docx 3.10 venv mirroring install_pico.sh. | PASS |
| 1b | 14 unit tests on bridge math: rotation correctness, palm-drop, frame anchor, axis mapping, distance preservation, shared-array roundtrip. | PASS (14/14) |
| 1c | 4 e2e tests with real dex_retargeting: YAML normalisation, retarget runs, response to input changes, motor-angle ranges within brainco spec. | PASS (4/4) |
| 1d | smoke_test_brainco.py: imports + asset wiring + bridge math, EXIT 0 on dev box. | PASS |
| 2 | Verify xrobotoolkit emits OpenXR-spec hand-joint data on real PICO firmware (5-line print script). | Pending hardware |
| 3 | Brainco hands powered, parked safe — verify warmup, mirror open/close, pinch. | Pending hardware |
| 4 | Full integrated PicoStreamer(enable_brainco=True) with arm IK. | Pending hardware |

## 5.1 Notable bug caught and fixed during validation

brainco.yml uses xr_teleoperate's union-style key naming (target_link_human_indices_dexpilot vs ..._vector). Current dex_retargeting (≥0.4) rejects suffixed keys with a TypeError. Without the fix, BraincoController() would have crashed on first construction. Resolved in hand_retargeting._normalize_section() and regression-guarded by test_retargeter_loads_with_brainco_yaml.

# 6. Deployment runbook

Run these stages in order. Do not skip ahead until each passes. Every command assumes you are at the repo root (Gear-sonic/GR00T-WholeBodyControl).

## 6.1 Install

bash install_scripts/install_pico.sh
source .venv_teleop/bin/activate

This sets up a Python 3.10 venv, installs gear_sonic[teleop] (which now includes dex-retargeting and pyyaml), and builds unitree_sdk2py and xrobotoolkit_sdk from external_dependencies/.

## 6.2 Stage 0 — hardware-free smoke test

python -m decoupled_wbc.control.teleop.main.smoke_test_brainco

All three stages should print [OK]. Exit 0 = ready to proceed. On the deployment box every line must be [OK]; if you see [DEP] lines, the runtime deps are not installed and you skipped step 6.1.

## 6.3 Stage 1 — unit tests

pytest decoupled_wbc/tests/control/teleop/brainco/

Should report 18 passed.

## 6.4 Stage 2 — PICO connected, brainco hands UNPOWERED

Goal: confirm xrobotoolkit emits OpenXR-spec hand-joint data on this firmware. Connect the PICO, launch the XRoboToolkit Unity app, enable hand tracking, then run:

python - <<'EOF'
import xrobotoolkit_sdk as xrt; xrt.init()
import time
while True:
    s = xrt.get_left_hand_tracking_state()
    print('palm_z=%.3f wrist_z=%.3f tip_index_z=%.3f' %
          (s[0][2], s[1][2], s[10][2]))
    time.sleep(0.5)
EOF

Hold your left hand at chest height with fingers up. Verify wrist_z < tip_index_z (fingertip is higher than wrist). If wrist_z > tip_index_z, the firmware uses a non-standard joint ordering and the spec assumption needs review before continuing.

## 6.5 Stage 3 — brainco hands powered, parked safe

Power the hands in a position where a sudden close cannot damage them or the robot.

python -m decoupled_wbc.control.teleop.main.run_brainco_teleop \
    --network-interface eth0

Expected log sequence:

xrobotoolkit SDK initialised.

Initialising BraincoController

[BraincoController] DDS state ready  (within 5 s, or TimeoutError fires)

BraincoController control process started

[BraincoController] warmup complete after 10 valid frames; publishing retargeted commands  (within ~100 ms of seeing your hand)

Then test interactively:

Hold hand still and open — brainco hand holds open.

Slowly close to a fist — brainco hand mirrors.

Pinch thumb-to-index — motor 0 (thumb) and motor 2 (index) close.

Move hand out of camera FOV — log shows tracking lost; reverting to open hand.

Move back in — warmup completes, retargeting resumes.

## 6.6 Stage 4 — full integrated teleop

Only after Stages 0–3 pass:

from decoupled_wbc.control.teleop.streamers.pico_streamer import PicoStreamer
streamer = PicoStreamer(enable_brainco=True)
# rest of your existing teleop main loop unchanged

Optional kwargs: brainco_fps (default 100), brainco_state_timeout_s (default 5.0), brainco_warmup_frames (default 10).

# 7. Failure modes & how to debug

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| TimeoutError: no DDS state on rt/brainco/... | Brainco hands not powered, or wrong DDS interface. | Confirm hands powered, on the same network. Check --network-interface. Or pass state_timeout_s=0 to wait forever. |
| Hand opens to safe pose then never closes | Warmup gate keeps resetting — transient is_active=0. | Move hand into camera FOV. If still failing, lower --warmup-frames (default 10) once you trust the source. |
| Brainco fingers move but inverted/rotated | Frame composition wrong. | Check R_HAND_TO_UNITREE in brainco_bridge.py against §3.2. Most likely fix is a sign flip on one column. |
| Single finger lags / never closes | Brainco firmware motor order differs from BraincoLeftJointIndex. | Compare against brainco-hz.com/docs/revolimb-hand/product/parameters.html. Adjust IntEnum in robot_hand_brainco.py. |
| _generate_finger_data falls back to button mode | xr_client.get_hand_tracking_state returned None (is_active=0). | Hand outside FOV or occlusion. Move into camera view. |
| Distance threshold never triggers in arm IK | Keypoints in non-meter units. | Print np.linalg.norm(positions[4] - positions[9]) at fist; should be ~3 cm. |
| ImportError: dex_retargeting | Teleop venv not installed or not active. | bash install_scripts/install_pico.sh && source .venv_teleop/bin/activate. |
| TypeError: ...target_link_human_indices_dexpilot... | Old hand_retargeting.py without _normalize_section. | Pull the latest brainco/hand_retargeting.py — this fix is in the deploy package. |

# 8. Quick reference

## 8.1 DDS topics

| Direction | Topic | Type |
| --- | --- | --- |
| Subscribe | rt/brainco/left/state | MotorStates_ |
| Subscribe | rt/brainco/right/state | MotorStates_ |
| Publish | rt/brainco/left/cmd | MotorCmds_ |
| Publish | rt/brainco/right/cmd | MotorCmds_ |

## 8.2 Brainco motor mapping

| Motor | Joint | Range (rad) |
| --- | --- | --- |
| 0 | thumb metacarpal | [0, 1.52] |
| 1 | thumb proximal (aux) | [0, 1.05] |
| 2 | index proximal | [0, 1.47] |
| 3 | middle proximal | [0, 1.47] |
| 4 | ring proximal | [0, 1.47] |
| 5 | pinky proximal | [0, 1.47] |

Published cmd values are normalised to [0, 1] by _normalize() — 0 = open, 1 = fully closed. Reference: brainco-hz.com/docs/revolimb-hand/product/parameters.html.

## 8.3 OpenXR hand-joint enum (XR_EXT_hand_tracking)

| Index | Joint | Index | Joint |
| --- | --- | --- | --- |
| 0 | PALM (dropped) | 13 | middle proximal |
| 1 | WRIST → kept index 0 | 14 | middle intermediate |
| 2 | thumb metacarpal | 15 | middle distal |
| 3 | thumb proximal | 16 | ring metacarpal |
| 4 | thumb distal | 17 | ring proximal |
| 5 | thumb tip → kept 4 | 18 | ring intermediate |
| 6 | index metacarpal | 19 | ring distal → kept 19 |
| 7 | index proximal | 20 | ring tip |
| 8 | index intermediate | 21 | little metacarpal |
| 9 | index distal | 22 | little proximal |
| 10 | index tip → kept 9 | 23 | little intermediate |
| 11 | middle metacarpal | 24 | little distal → kept 24 |
| 12 | middle (skip) | 25 | little tip |

After dropping PALM, the bridge produces a (25, 3) cloud whose wrist sits at index 0 and fingertips at 4 / 9 / 14 / 19 / 24 — matching the indices referenced by brainco.yml.

# 9. Pre-deployment sign-off checklist

Smoke test passes on the target machine (Stage 0).

All 18 unit + e2e tests pass on the target machine (Stage 1).

PICO Stage-2 print script confirms wrist_z < tip_index_z.

Stage 3 standalone runner mirrors the operator's hand with no 90°-off rotation.

Tracking-loss revert log line observed when hand leaves FOV.

Warmup-complete log line observed within ~150 ms of hand re-entering FOV.

PicoStreamer(enable_brainco=True) integrated test passes; arm IK pipeline unchanged.

Operator briefed: hand commands route over rt/brainco/{left,right}/cmd; existing arm IK observes real fingertip distances now (5 cm pinch threshold).

Document generated by scripts/generate_brainco_deploy_doc.py. If anything in this guide drifts from the source, regenerate.